S7U4_2

Q. Enzyme Technology: Industrial enzymes, immobilization techniques, and applications in food, textile, and pharmaceutical industries.

Enzyme technology is the application of enzymes, particularly microbial enzymes, for the production of useful products and for improving industrial processes. Enzymes are biological catalysts that accelerate biochemical reactions without being consumed during the reaction. Microorganisms such as Bacillus, Aspergillus, Trichoderma, Saccharomyces and Candida are important sources of industrial enzymes because they grow rapidly and can produce enzymes in large quantities.

1. Industrial Enzymes

Some important industrial enzymes include:

| Enzyme | Common microbial source | Major application |
| --- | --- | --- |
| Amylase | Bacillus, Aspergillus | Starch processing, baking |
| Protease | Bacillus, Aspergillus | Detergents, food processing |
| Lipase | Candida, Bacillus | Dairy, detergents, pharmaceuticals |
| Cellulase | Trichoderma, Aspergillus | Textile processing, biofuel |
| Pectinase | Aspergillus | Fruit juice clarification |
| Lactase | Kluyveromyces | Lactose-free milk |
| Xylanase | Bacillus, fungi | Pulp and paper, food processing |

2. Enzyme Immobilization

Enzyme immobilization is the process of restricting enzymes to a particular region or solid support while retaining their catalytic activity. Immobilized enzymes can be separated easily from the reaction mixture and reused.

Major immobilization techniques

Other techniques include cross-linking and encapsulation.

Advantages of immobilized enzymes

- Enzymes can be recovered and reused.

- Improved stability under suitable conditions.

- Product separation becomes easier.

- Suitable for continuous industrial processes.

- Can reduce production costs.

3. Applications of Enzyme Technology

A. Food Industry

- Enzymes are extensively used in food processing.

- Amylases convert starch into sugars and improve bread quality.

- Pectinases clarify fruit juices and increase juice extraction.

- Lactase hydrolyses lactose to produce lactose-free dairy products.

- Proteases are used in cheese production and protein modification.

- Lipases contribute to flavour development in certain dairy products.

B. Textile Industry

- Enzymes provide relatively selective and environmentally favourable alternatives to some conventional chemical treatments.

- Cellulases are used for biopolishing of cotton fabrics and denim finishing.

- Amylases remove starch-based sizing materials from fabrics.

- Pectinases assist in bioscouring of cotton.

- Laccases can be used in textile dye modification and finishing.

C. Pharmaceutical Industry

- Enzymes are important both as manufacturing tools and as therapeutic or diagnostic agents.

- Enzymes are used in the synthesis and modification of pharmaceutical compounds.

- Penicillin acylase is used in the production of important β-lactam antibiotic intermediates.

- Proteolytic enzymes have applications in pharmaceutical processing.

- Enzymes such as asparaginase have therapeutic applications.

- Enzyme-based assays are widely used for diagnosis and biochemical analysis.

Enzyme technology combines microbiology, biochemistry and industrial biotechnology to obtain useful products through efficient and controlled biological catalysis. The use of immobilized enzymes is particularly important because it permits enzyme recovery, reuse and continuous processing.