S7U4_3

Q. Bioremediation of Pollutants: Microbial degradation of hydrocarbons, heavy metals, pesticides, and plastics.

Bioremediation is the use of living organisms, particularly microorganisms, to degrade, transform, detoxify, or remove pollutants from contaminated soil, water, and other environmental systems. Microorganisms such as bacteria, fungi, algae, and archaea possess diverse metabolic pathways that enable them to utilize or transform many environmental contaminants.

Basic principle

Polluted environment

↓

Selection of suitable microorganisms

↓

Microbial growth and metabolic activity

↓

Degradation / transformation of pollutant

↓

Less toxic or harmless products

↓

Environmental recovery

1. Microbial Degradation of Hydrocarbons

Hydrocarbons are major pollutants associated with petroleum spills, refineries, automobile activities and industrial wastes. Several microorganisms can use hydrocarbons as carbon and energy sources.

Examples include:

- Pseudomonas

- Bacillus

- Rhodococcus

- Alcanivorax

- Aspergillus

- Candida

Microorganisms produce enzymes such as oxygenases and hydroxylases that initiate the breakdown of hydrocarbons.

Hydrocarbon → Alcohol → Aldehyde → Organic acid → CO₂ + H₂O

Hydrocarbon biodegradation is particularly important in the remediation of oil-contaminated soil and marine environments.

2. Bioremediation of Heavy Metals

Heavy metals such as lead (Pb), mercury (Hg), cadmium (Cd), chromium (Cr), arsenic (As), and nickel (Ni) cannot be destroyed by biodegradation because they are elements. However, microorganisms can transform, immobilize, precipitate, or accumulate them, thereby reducing their mobility or toxicity.

Major microbial mechanisms

- Biosorption: binding of metal ions to microbial cell surfaces.

- Bioaccumulation: uptake and storage of metals inside cells.

- Biotransformation: conversion of metals into different chemical forms.

- Bioprecipitation: conversion into less soluble compounds.

- Bioreduction/oxidation: alteration of the oxidation state and consequently the mobility or toxicity of certain metals.

For example, microbial reduction of Cr(VI) to Cr(III) can reduce chromium mobility and toxicity under appropriate environmental conditions.

3. Microbial Degradation of Pesticides

Pesticides can persist in soil and water and may affect non-target organisms. Certain bacteria and fungi possess enzymes capable of degrading pesticide molecules.

Important microbial genera include:

- Pseudomonas

- Bacillus

- Flavobacterium

- Sphingomonas

- Aspergillus

- Trichoderma

Microbial degradation may involve:

Hydrolysis → Oxidation/reduction → Ring cleavage → Mineralization

The final products may include simpler compounds such as CO₂, H₂O, inorganic nutrients and microbial biomass, although the exact products depend on the pesticide and pathway.

4. Microbial Degradation of Plastics

Conventional plastics are highly resistant to natural degradation because of their complex chemical structures and high molecular weight. However, some microorganisms can colonize and partially degrade certain plastics.

Microorganisms reported to have plastic-degrading potential include bacteria belonging to genera such as Pseudomonas, Bacillus and Ideonella, as well as several fungi.

General process

Plastic surface

↓

Microbial attachment

↓

Biofilm formation

↓

Enzyme secretion

↓

Polymer breakdown

↓

Smaller molecules

↓

Further microbial metabolism

↓

CO₂ + H₂O + biomass

A well-known example is PET degradation by Ideonella sakaiensis, which produces enzymes capable of breaking down polyethylene terephthalate (PET) into smaller compounds.

Important: Plastic biodegradation is generally slow and varies greatly with the type of polymer and environmental conditions. Complete mineralization should not be assumed for all plastics.

5. Factors Affecting Bioremediation

The efficiency of microbial bioremediation depends on:

- Temperature

- pH

- Moisture

- Oxygen availability

- Nutrient availability

- Pollutant concentration

- Microbial population

- Nature and bioavailability of the pollutant

- Presence of suitable electron donors and acceptors

Key concept

Suitable microorganisms + suitable environmental conditions → Effective bioremediation

Summary Table

| Pollutant | Microbial action | Major mechanism |
| --- | --- | --- |
| Hydrocarbons | Breakdown into simpler compounds | Oxidation and mineralization |
| Heavy metals | Transformation or immobilization | Biosorption, bioaccumulation, reduction, precipitation |
| Pesticides | Degradation and transformation | Hydrolysis, oxidation, reduction |
| Plastics | Partial polymer degradation | Biofilm formation and extracellular enzymes |