S7U4_4

Q. Microbial Biosensors for Environmental Monitoring: Principles, types, and applications for detecting pollutants.

Microbial biosensors are analytical devices that use living microorganisms or microbial components as the biological sensing element to detect and measure specific substances in environmental samples. They combine the biological specificity of microorganisms with a physical or chemical transducer to provide rapid information about pollutants.

1. Principle of Microbial Biosensors

A microbial biosensor generally consists of three main components:

Biological sensing element – microorganism, microbial enzyme, or whole microbial cell.

Transducer – converts the biological response into an electrical, optical, thermal, or other measurable signal.

Signal-processing system – amplifies and displays the signal.

Basic principle

Environmental sample

↓

Target pollutant

↓

Microbial sensing element

↓

Biological response

(growth / respiration / enzyme activity)

↓

Transducer

↓

Electrical / optical signal

↓

Signal processing

↓

Pollutant detection

For example, when a pollutant affects the respiration of a microorganism, the resulting change in oxygen consumption or carbon dioxide production can be measured and related to pollutant concentration.

Types of Microbial Biosensors

Microbial biosensors can be classified according to the type of biological element and transducer used.

A. Whole-cell microbial biosensors

Living microbial cells are directly used as the sensing element.

Examples: Escherichia coli, Pseudomonas, yeast and other suitable microorganisms.

They are useful for detecting substances that affect microbial metabolism or respiration.

B. Enzyme-based microbial biosensors

Specific microbial enzymes are immobilized on a sensor surface. The target compound causes a change in enzyme activity or generates a measurable reaction product.

C. Genetically engineered microbial biosensors

Microorganisms are genetically modified so that exposure to a particular pollutant produces a measurable response, such as bioluminescence or fluorescence.

Classification according to transducer

| Type | Signal measured | Typical use |
| --- | --- | --- |
| Electrochemical | Current, voltage, conductivity | Organic pollutants, toxic chemicals |
| Optical | Fluorescence, colour, bioluminescence | Toxic compounds, metals |
| Respirometric | O₂ consumption/CO₂ production | Biodegradable organic matter |
| Thermal | Heat generated/absorbed | Metabolic reactions |

3. Detection of Environmental Pollutants

Microbial biosensors can be designed to detect several groups of pollutants.

Heavy metals

Metals such as mercury, cadmium, lead, arsenic and chromium can interfere with microbial metabolism. Sensitive microorganisms can therefore be used to indicate their presence.

Heavy metal

↓

Microbial cell

↓

Interaction with cellular components

↓

Change in metabolic/gene response

↓

Measurable signal

↓

Metal detected

Organic pollutants

Microbial biosensors can be used for detecting or assessing:

- Phenols

- Hydrocarbons

- Pesticides

- Aromatic compounds

- Industrial organic chemicals

Biological oxygen demand (BOD)

Respirometric microbial biosensors can estimate biodegradable organic pollution by measuring microbial oxygen consumption.

A higher concentration of biodegradable organic matter generally produces greater microbial respiration and oxygen consumption.

Applications in Environmental Monitoring

Microbial biosensors have several important applications:

Water quality monitoring

They can be used to detect toxic chemicals, organic pollutants and heavy metals in surface water and wastewater.

Industrial wastewater

Biosensors can provide rapid screening for toxic substances before wastewater is discharged.

Soil pollution

They can help assess the biological toxicity or bioavailability of contaminants in contaminated soils.

Pesticide monitoring

Specific microbial responses can be used for detecting pesticide residues and their biological effects.

Oil and hydrocarbon pollution

Microorganisms capable of responding to hydrocarbons can be incorporated into biosensors for monitoring petroleum contamination.

Advantages and Limitations

Advantages

- Rapid detection

- Relatively simple operation

- High biological specificity

- Can detect biological effects, not merely chemical concentration

- Potential for continuous or on-site monitoring

- May be less expensive than some conventional analytical methods

Limitations

- Microorganisms can be sensitive to temperature and pH.

- Other substances in environmental samples may interfere with the response.

- Biosensor stability can decrease over time.

- Quantification may require calibration against standard analytical methods.

- A biological response does not always identify the exact chemical responsible.

6. Microbial Biosensors vs Conventional Methods

| Feature | Microbial biosensor | Conventional chemical analysis |
| --- | --- | --- |
| Detection | Biological response | Chemical measurement |
| Speed | Often rapid | May require longer laboratory procedures |
| Specificity | Depends on organism/design | Often highly specific |
| Equipment | Relatively simple in some formats | Often sophisticated |
| Information obtained | Toxicity/bioavailability or target response | Concentration/chemical identity |
| Field application | Potentially high | Often laboratory-based |

Chemical analysis tells us “how much pollutant is present,” whereas a microbial biosensor can also provide information about “how the pollutant affects a biological system.”

Microbial biosensors are important tools in environmental biotechnology because they combine the sensitivity of biological systems with rapid instrumental detection. They can be used for monitoring heavy metals, pesticides, hydrocarbons, organic pollutants and overall biological toxicity, making them valuable complements to conventional environmental analytical techniques.